PHOENIX NEST LLC

LOG SPLITTER

AUTO EXTEND / RETURN

INSTALLATION & WIRING MANUAL

| Drawing | PN-LS-002 Rev A |
| --- | --- |
| System | 12VDC — 2x DPDT DIN Rail Relay |
| Valve | Double Solenoid (SOL-A Extend / SOL-B Retract) |
| Controls | E-Stop, FWD Jog, BWD Jog, Cycle Auto |
| Date | February 2026 |

KEEP THIS MANUAL WITH THE EQUIPMENT AT ALL TIMES

# TABLE OF CONTENTS

1. System Overview	3

2. Wire Schedule & Label Chart	4

3. DIN Rail Relay Pinout	6

4. Installation — Phase 1: Mounting	7

5. Installation — Phase 2: Power Circuit	8

6. Installation — Phase 3: Manual Jog Circuits	9

7. Installation — Phase 4: K1 Cycle Latch	10

8. Installation — Phase 5: K2 Direction	11

9. Installation — Phase 6: K2 Solenoid Outputs & Full Test	12

10. Troubleshooting	13

11. Optional Modifications	14

12. Wiring Schematic (Reference)	15

# 1. SYSTEM OVERVIEW

This system adds automatic extend-and-return functionality to a hydraulic log splitter using relay logic. One press of the CYCLE button extends the cylinder, splits the wood, and automatically retracts to the home position.

OPERATING MODES

Manual Jog (FWD): Hold FWD button to extend cylinder. Release to stop. Direct-wired to SOL-A, bypasses all relay logic.

Manual Jog (BWD): Hold BWD button to retract cylinder. Release to stop. Direct-wired to SOL-B, bypasses all relay logic.

Auto Cycle: Press CYCLE once. Cylinder extends to full stroke (or until LS1 trips), then automatically retracts to home (LS2). One button press, hands-free return.

E-Stop: Kills all power immediately. Twist to release. Cycle must be restarted manually.

HOW THE AUTO CYCLE WORKS

K1 (Cycle Latch): When CYCLE is pressed, K1 energizes. K1-1 contact seals (latches) so K1 stays on after you release the button. K1 stays latched until LS2 opens (cylinder reaches home).

K2 (Direction): K1-2 feeds power through LS1 to K2 coil. When K2 is energized, its changeover contact routes +12V to SOL-A (extend). When LS1 opens (full extend), K2 drops, and the changeover flips to SOL-B (retract).

Key insight: K2's DPDT changeover guarantees only one solenoid is ever powered during auto cycle. It is mechanically impossible for both to be on simultaneously.

SAFETY FEATURES

E-Stop kills entire +12V bus upstream of everything. Manual jog buttons bypass relay logic entirely — if a relay fails, you can still move the cylinder. Both limit switches are NC (normally closed) — a broken wire is the same as the switch being open, which stops the cycle (fail-safe).

# 2. WIRE SCHEDULE & LABEL CHART

Label both ends of every wire with heat-shrink markers before installing. This chart is your wiring bible.

WIRE LABEL FORMAT: Print the Wire ID (W01, W02, etc.) on heat-shrink tubing. Slide onto wire BEFORE crimping terminals. Label goes on BOTH ends.

| ID | DESCRIPTION | COLOR | GAUGE | FROM → TO |
| --- | --- | --- | --- | --- |
| W01 | +12V BATT to FUSE | RED | 12 AWG | Battery + terminal → Fuse input |
| W02 | FUSE to E-STOP | RED | 12 AWG | Fuse output → E-Stop NC terminal (C1) |
| W03 | E-STOP to BUS (A) | RED | 12 AWG | E-Stop NC terminal (C2) → Bus junction A on DIN terminal block |
| W04 | BUS (A) to FWD btn | RED | 14 AWG | Bus A → FWD button terminal 1 |
| W05 | FWD btn to SOL-A | RED | 14 AWG | FWD button terminal 2 → SOL-A coil terminal (+) |
| W06 | SOL-A to GND | BLACK | 14 AWG | SOL-A coil terminal (−) → Ground bus |
| W07 | BUS (A) to BWD btn | RED | 14 AWG | Bus A → BWD button terminal 1 |
| W08 | BWD btn to SOL-B | RED | 14 AWG | BWD button terminal 2 → SOL-B coil terminal (+) |
| W09 | SOL-B to GND | BLACK | 14 AWG | SOL-B coil terminal (−) → Ground bus |
| W10 | BUS (A) to CYCLE btn | RED | 14 AWG | Bus A → CYCLE button terminal 1 |
| W11 | CYCLE btn to junction B | RED | 16 AWG | CYCLE button terminal 2 → Junction B |
| W12 | BUS (A) to K1-1 COM | RED | 16 AWG | Bus A → K1 relay pole 1 common (pin 1) |
| W13 | K1-1 NO to junction B | RED | 16 AWG | K1 pole 1 NO (pin 4) → Junction B (seal path) |
| W14 | Junction B to LS2 | RED | 16 AWG | Junction B → LS2 NC terminal 1 |
| W15 | LS2 to K1 COIL (+) | RED | 16 AWG | LS2 NC terminal 2 → K1 coil terminal (+) (pin 2) |
| W16 | K1 COIL (−) to GND | BLACK | 16 AWG | K1 coil terminal (−) (pin 7) → Ground bus |
| W17 | BUS (A) to K1-2 COM | RED | 14 AWG | Bus A → K1 relay pole 2 common (pin 8) |
| W18 | K1-2 NO to LS1 | RED | 16 AWG | K1 pole 2 NO (pin 5) → LS1 NC terminal 1 |
| W19 | LS1 to K2 COIL (+) | RED | 16 AWG | LS1 NC terminal 2 → K2 coil terminal (+) (pin 2) |
| W20 | K2 COIL (−) to GND | BLACK | 16 AWG | K2 coil terminal (−) (pin 7) → Ground bus |
| W21 | BUS (A) to K2-1 COM | RED | 14 AWG | Bus A → K2 relay pole 1 common (pin 1) |
| W22 | K2-1 NO to SOL-A (+) | GREEN | 14 AWG | K2 pole 1 NO (pin 4) → SOL-A coil terminal (+) |
| W23 | K2-1 NC to SOL-B (+) | GREEN | 14 AWG | K2 pole 1 NC (pin 3) → SOL-B coil terminal (+) |
| W24 | BATT (−) to GND BUS | BLACK | 12 AWG | Battery − terminal → Ground bus on DIN terminal block |
| W25 | GND BUS to CHASSIS | BLACK | 12 AWG | Ground bus → Splitter frame bolt (clean metal) |

NOTE: W22 and W23 (green wires from K2 to solenoids) connect to the SAME solenoid terminals as W05 and W08 (red jog wires). Each solenoid has two hot wires feeding it — one from the jog button (direct) and one from K2 (auto cycle). This is by design.

# 3. DIN RAIL RELAY PINOUT

Standard 8-pin DIN rail DPDT relay pinout. Verify against your specific relay datasheet — pin numbers may vary by manufacturer. The pinout below is for the most common configuration.

STANDARD 8-PIN DPDT RELAY

| PIN | FUNCTION | K1 (CYCLE LATCH) | K2 (DIRECTION) |
| --- | --- | --- | --- |
| 1 | Pole 1 — COM | W12: +12V bus (A) | W21: +12V bus (A) |
| 2 | Coil (+) | W15: from LS2 | W19: from LS1 |
| 3 | Pole 1 — NC | (not used) | W23: to SOL-B (+) |
| 4 | Pole 1 — NO | W13: to junction B (seal) | W22: to SOL-A (+) |
| 5 | Pole 2 — NO | W18: to LS1 | (not used) |
| 6 | Pole 2 — NC | (not used) | (not used) |
| 7 | Coil (−) | W16: to Ground bus | W20: to Ground bus |
| 8 | Pole 2 — COM | W17: +12V bus (A) | (not used) |

DIODE PLACEMENT: D3 across K1 pins 2 (+) and 7 (−). D4 across K2 pins 2 (+) and 7 (−). Cathode band (stripe) toward pin 2 on both.

VERIFY YOUR RELAY: Before wiring, use a multimeter in continuity mode. Apply 12V to pins 2 and 7. Check that pin 1 connects to pin 4 (NO closes) and pin 8 connects to pin 5 (NO closes). If your relay is different, adjust pin numbers throughout this manual.

# 4. INSTALLATION — PHASE 1: MOUNTING

Step 1.1: Mount DIN rail on mounting plate. Secure plate to splitter frame with bolts and lock washers. Choose a location protected from rain, hydraulic spray, and heat.

Step 1.2: Mount K1 relay socket on DIN rail (left position).

Step 1.3: Mount K2 relay socket on DIN rail (right position).

Step 1.4: Mount DIN-rail terminal blocks between relay sockets. You need at least 3 blocks: one for +12V bus (A), one for Ground bus, and one for junction B.

Step 1.5: Mount inline fuse holder near battery connection point.

Step 1.6: Mount operator panel box on splitter frame — accessible from operating position. Drill holes for E-STOP, FWD, BWD, and CYCLE buttons. E-STOP should be largest and most prominent.

Step 1.7: Install all pushbuttons in panel box. E-STOP at top, CYCLE below it, FWD and BWD at bottom.

Step 1.8: Mount LS1 at the full-extend position. The cylinder rod, wedge, or pusher plate should activate it at maximum stroke.

| ⚠ | Manually push cylinder to full extend. Verify LS1 clicks. You should hear/feel it actuate. |
| --- | --- |

Step 1.9: Mount LS2 at the full-retract (home) position. The cylinder rod should activate it when fully retracted.

| ⚠ | Manually pull cylinder to full retract. Verify LS2 clicks. |
| --- | --- |

# 5. INSTALLATION — PHASE 2: POWER CIRCUIT

Step 2.1: DISCONNECT BATTERY. Remove negative terminal cable from battery first.

Step 2.2: Run wire W24 (BLACK 12AWG) from battery negative terminal to Ground bus terminal block. Do NOT connect to battery yet.

Step 2.3: Run wire W25 (BLACK 12AWG) from Ground bus to clean bare metal on splitter frame. Use star washer under bolt head for good contact.

Step 2.4: Run wire W01 (RED 12AWG) from battery positive terminal to fuse holder input. Do NOT connect to battery yet.

Step 2.5: Run wire W02 (RED 12AWG) from fuse holder output to E-STOP NC terminal (C1).

Step 2.6: Run wire W03 (RED 12AWG) from E-STOP NC terminal (C2) to +12V bus (A) terminal block.

Step 2.7: Install 15A fuse in fuse holder.

Step 2.8: Connect battery cables — negative first, then positive.

Step 2.9: TEST: Using a multimeter, measure voltage at +12V bus (A) terminal block. Should read ~12.5V with E-STOP in normal (released) position.

| ⚠ | EXPECTED: ~12.5V at bus A. Press E-STOP — voltage should drop to 0V. Twist-release E-STOP — voltage returns. If no voltage: check fuse, check battery connections, check E-STOP wiring. |
| --- | --- |

# 6. INSTALLATION — PHASE 3: MANUAL JOG CIRCUITS

Step 3.1: Run wire W04 (RED 14AWG) from +12V bus (A) to FWD button terminal 1.

Step 3.2: Run wire W05 (RED 14AWG) from FWD button terminal 2 to SOL-A coil (+) terminal.

Step 3.3: Run wire W06 (BLACK 14AWG) from SOL-A coil (−) terminal to Ground bus.

Step 3.4: Install flyback diode D1 (1N4007) across SOL-A coil terminals. CATHODE BAND TOWARD (+) TERMINAL.

Step 3.5: TEST: Start the hydraulic power unit (engine/PTO). Press and hold FWD button.

| ⚠ | EXPECTED: Cylinder extends while FWD is held. Release FWD — cylinder stops. If nothing happens: check W04, W05, W06 connections. Verify SOL-A clicks when FWD pressed (listen/feel). Check diode orientation — reversed diode = dead short. |
| --- | --- |

Step 3.6: Run wire W07 (RED 14AWG) from +12V bus (A) to BWD button terminal 1.

Step 3.7: Run wire W08 (RED 14AWG) from BWD button terminal 2 to SOL-B coil (+) terminal.

Step 3.8: Run wire W09 (BLACK 14AWG) from SOL-B coil (−) terminal to Ground bus.

Step 3.9: Install flyback diode D2 (1N4007) across SOL-B coil terminals. CATHODE BAND TOWARD (+) TERMINAL.

Step 3.10: TEST: Press and hold BWD button.

| ⚠ | EXPECTED: Cylinder retracts while BWD is held. Release BWD — cylinder stops. If nothing happens: same checks as 3.5 but for SOL-B. |
| --- | --- |

Step 3.11: TEST: Use FWD and BWD to jog the cylinder through its full stroke. Verify LS1 clicks at full extend and LS2 clicks at full retract.

| ⚠ | EXPECTED: Both limit switches actuate at correct positions. If a switch doesn't actuate: adjust mounting position. This is critical — auto cycle depends on both switches. |
| --- | --- |

# 7. INSTALLATION — PHASE 4: K1 CYCLE LATCH CIRCUIT

Step 4.1: Insert K1 relay into its DIN rail socket. Note pin orientation — pin 1 is usually marked on the socket.

Step 4.2: Install flyback diode D3 (1N4007) across K1 coil pins (pins 2 and 7). CATHODE BAND TOWARD PIN 2 (+).

Step 4.3: Run wire W10 (RED 14AWG) from +12V bus (A) to CYCLE button terminal 1.

Step 4.4: Run wire W11 (RED 16AWG) from CYCLE button terminal 2 to junction B terminal block.

Step 4.5: Run wire W12 (RED 16AWG) from +12V bus (A) to K1 pole 1 common (pin 1).

Step 4.6: Run wire W13 (RED 16AWG) from K1 pole 1 NO (pin 4) to junction B terminal block.

Step 4.7: Run wire W14 (RED 16AWG) from junction B to LS2 terminal 1.

Step 4.8: Run wire W15 (RED 16AWG) from LS2 terminal 2 to K1 coil (+) (pin 2).

Step 4.9: Run wire W16 (BLACK 16AWG) from K1 coil (−) (pin 7) to Ground bus.

Step 4.10: Use BWD button to jog cylinder to the HOME position (full retract). LS2 must be in its normal (closed) state.

Step 4.11: TEST: Press CYCLE button momentarily and release.

| ⚠ | EXPECTED: K1 relay clicks ON when CYCLE pressed, and STAYS ON after you release the button (seal contact is holding it). K1 should remain latched. If K1 drops when you release CYCLE: check W12, W13 (seal path). Verify pin 1 and pin 4 on K1 socket. |
| --- | --- |

Step 4.12: TEST: With K1 still latched, use FWD to jog cylinder away from home. Then use BWD to jog it back until LS2 clicks.

| ⚠ | EXPECTED: When LS2 actuates (cylinder reaches home), K1 should UNLATCH (click off). This confirms the auto-stop at home works. If K1 doesn't drop: check W14, W15 (LS2 path). Verify LS2 is NC type and wired correctly. |
| --- | --- |

# 8. INSTALLATION — PHASE 5: K2 DIRECTION CIRCUIT

Step 5.1: Insert K2 relay into its DIN rail socket.

Step 5.2: Install flyback diode D4 (1N4007) across K2 coil pins (pins 2 and 7). CATHODE BAND TOWARD PIN 2 (+).

Step 5.3: Run wire W17 (RED 14AWG) from +12V bus (A) to K1 pole 2 common (pin 8).

Step 5.4: Run wire W18 (RED 16AWG) from K1 pole 2 NO (pin 5) to LS1 terminal 1.

Step 5.5: Run wire W19 (RED 16AWG) from LS1 terminal 2 to K2 coil (+) (pin 2).

Step 5.6: Run wire W20 (BLACK 16AWG) from K2 coil (−) (pin 7) to Ground bus.

Step 5.7: TEST: Jog cylinder to HOME with BWD. Press CYCLE to latch K1.

| ⚠ | EXPECTED: Both K1 AND K2 should click ON. K2 energizes because K1-2 closes (pin 8→5), feeding through LS1 (closed at this point) to K2 coil. If K1 latches but K2 doesn't: check W17, W18, W19, W20. Verify K1 pin 8 (COM) and pin 5 (NO). |
| --- | --- |

# 9. INSTALLATION — PHASE 6: K2 SOLENOID OUTPUTS

Step 6.1: Run wire W21 (RED 14AWG) from +12V bus (A) to K2 pole 1 common (pin 1).

Step 6.2: Run wire W22 (GREEN 14AWG) from K2 pole 1 NO (pin 4) to SOL-A coil (+) terminal.

Step 6.3: Run wire W23 (GREEN 14AWG) from K2 pole 1 NC (pin 3) to SOL-B coil (+) terminal.

Step 6.4: NOTE: SOL-A and SOL-B ground wires (W06, W09) are already connected from Phase 3. The solenoids now have two possible power sources each — direct from jog buttons, and switched through K2.

Step 6.5: FULL SYSTEM TEST: Jog cylinder to HOME position with BWD. Place a piece of wood on the splitter bed.

Step 6.6: Press CYCLE button once and release.

| ⚠ | EXPECTED FULL SEQUENCE: 1. K1 clicks ON (latches) 2. K2 clicks ON (direction = extend) 3. SOL-A fires — cylinder extends toward wood 4. Cylinder hits wood, splits it, continues to LS1 5. LS1 actuates — K2 clicks OFF 6. SOL-B fires — cylinder retracts automatically 7. Cylinder reaches home — LS2 actuates 8. K1 clicks OFF — cycle complete, all quiet Total cycle should be one smooth extend-split-return motion with a single button press. |
| --- | --- |

Step 6.7: TEST E-STOP: Start another cycle. While cylinder is extending, press E-STOP.

| ⚠ | EXPECTED: Everything stops immediately. Cylinder holds position. Twist-release E-STOP. System should be dead — K1 unlatched when power was cut. Jog cylinder home with BWD. Start a new cycle to confirm normal operation resumes. |
| --- | --- |

Step 6.8: TEST MANUAL OVERRIDE: During auto cycle, try pressing FWD and BWD. Both jog buttons should still function (they bypass relay logic). This is your backup if a relay fails.

| ⚠ | EXPECTED: Jog buttons work regardless of cycle state. OPTIONAL: If you want to DISABLE jog during auto cycle (interlock), see Optional Modifications section. |
| --- | --- |

# 10. TROUBLESHOOTING

| SYMPTOM | LIKELY CAUSE | CHECK / FIX |
| --- | --- | --- |
| Cylinder won't extend (FWD button) | No power at bus A | Check fuse. Check E-STOP. Measure 12V at bus A. |
|  | SOL-A not clicking | Check W04, W05. Listen for solenoid click. Swap SOL-A/SOL-B wires briefly to test if solenoid is dead. |
|  | Diode D1 reversed | Reversed diode = dead short. Fuse will blow. Check cathode band orientation. |
| Cylinder won't retract (BWD button) | Same as above | Same checks but for W07, W08, W09 and SOL-B / D2. |
| CYCLE button — nothing happens | K1 not energizing | Check W10, W11. Verify junction B connections. Measure voltage at K1 coil pins with CYCLE held. |
|  | LS2 is open (not at home) | Jog cylinder to home with BWD until LS2 clicks. LS2 must be closed for K1 to latch. |
| K1 latches but drops immediately | Seal contact not working | Check W12, W13. Verify K1 pin 1 (COM) and pin 4 (NO). Check junction B has continuity. |
| K1 latches, cylinder extends, but won't auto-retract | LS1 not actuating | Jog to full extend with FWD. Verify LS1 clicks. Adjust LS1 position if needed. |
|  | K2 not switching | Measure voltage at K2 coil with K1 latched. Check W17, W18, W19, W20. |
| Cylinder retracts but doesn't stop at home | LS2 not actuating | Jog slowly to home with BWD. Verify LS2 clicks. Adjust LS2 position. |
|  | LS2 wired NO instead of NC | LS2 must be NC (closed at rest, opens when actuated). Check switch type. |
| Fuse keeps blowing | Short circuit | Disconnect all loads. Reconnect one at a time. Check for pinched wires, reversed diodes, or bare wire touching frame. |
| Relay chatters / buzzes | Low voltage | Check battery charge. Check for voltage drop under load (corroded connections). |
|  | Bad ground | Clean ground connection at frame. Check W24, W25. |
| Solenoid hums but cylinder doesn't move | Hydraulic issue, not electrical | Check hydraulic fluid level. Check for air in lines. Verify PTO/engine RPM. |

# 11. OPTIONAL MODIFICATIONS

JOG INTERLOCK (Disable FWD/BWD During Auto Cycle)

To prevent the operator from pressing FWD or BWD while an auto cycle is running, route the jog button power through a K1 NC contact. When K1 is latched (auto cycle active), the NC contact opens, disabling jog.

How: Disconnect W04 and W07 from +12V bus (A). Instead, connect them to K1 pole 2 NC (pin 6). Run a new wire from +12V bus (A) to K1 pole 2 COM (pin 8) — this wire may already exist as W17. When K1 is off (no auto cycle), pin 8 connects to pin 6, feeding the jog buttons. When K1 latches, pin 6 disconnects.

Trade-off: Jog buttons won't work during auto cycle. If a relay fails mid-cycle, you lose manual override until you hit E-Stop and K1 drops.

SAFETY TIMER (Auto-Kill on Timeout)

If LS1 never trips (jammed log, broken switch), the cylinder will push against the relief valve indefinitely. A time-delay relay can kill the cycle after a set time.

How: Add a 12VDC time-delay relay (TON, 15-20 seconds) with its coil powered by the same circuit as K1. Wire the timer's NC contact in series with K1 coil (between LS2 and K1 pin 2). When the timer expires, its NC contact opens, dropping K1 and killing the cycle. Set the timer to your cylinder's full stroke time plus a few seconds of margin.

CYCLE INDICATOR LIGHT

How: Wire a 12V LED indicator from K1 pole 2 NO (pin 5) to ground through a 1K resistor. LED illuminates whenever K1 is latched (auto cycle active). Use the E-Stop NO contact (spare) to drive a separate red LED that illuminates when E-Stop is pressed.

PRESSURE SWITCH INSTEAD OF LS1

A pressure switch can replace LS1 for more reliable split detection. Set it approximately 200 PSI below your system relief valve pressure. The switch triggers when backpressure rises (wood splits or cylinder stalls), regardless of where the cylinder is in its stroke. This means short pieces of wood don't require full cylinder extension.

Wiring: Identical to LS1. NC pressure switch in series with K2 coil. Opens when pressure threshold is reached.

# 12. WIRING SCHEMATIC (REFERENCE)

See the separate printed schematic (DWG PN-LS-002) in this binder. The schematic shows all circuits in standard wiring diagram format. Use it for tracing wires when troubleshooting.

The schematic and this installation manual describe the same system. Wire IDs (W01–W25) on the schematic match this manual exactly.

CIRCUIT SUMMARY

| CIRCUIT | WIRE PATH |
| --- | --- |
| Power | BATT (+) → FUSE → E-STOP NC → +12V Bus (A) |
| Manual FWD | Bus (A) → FWD btn → SOL-A → GND |
| Manual BWD | Bus (A) → BWD btn → SOL-B → GND |
| K1 Latch | Bus (A) → [CYCLE btn ¦¦ K1-1 seal] → LS2 NC → K1 coil → GND |
| K2 Direction | Bus (A) → K1-2 NO → LS1 NC → K2 coil → GND |
| K2 Outputs | Bus (A) → K2-1 COM → NO=SOL-A / NC=SOL-B → GND |